[image: image1.wmf]0

V

UNIVERSITY OF THE FREE STATE
DEPARTMENT OF PHYSICS
B.Sc. Test: October 2007
FSK 224 – Electronics

ASSESSOR

:  Prof. JJ Terblans

MODERATOR
:  Prof. WD Roos

TIME:  [image: image20.wmf]1

 HOUR
MARKS: [50]


Question 1

1.1
The colour of an LED is determined by?


A) The operating voltage.

B) The type of semiconductor material used in the LED's construction. 

C) The operating current.

D) The value of resistance inserted in series with the LED 

(2)

1.2
Which of the following electronic components are a "current controlled" device?


A) Rheostat 

B) Diode 
C) Coil or Inductor 
D) Transistor 

(2)

1.3
During forward-bias would a voltage drop of 0.3 V between the base and the emitter indicate that the transistor is:

A) Open. 

B) Germanium.
C) Silicon.
D) Shorted. 

(2)

1.4
Which terminal on an Op-amp schematic is indicated with a (–) sign?

A) Inverting input. 
B) Non-inverting input. 
C) –Vcc. 
D) Output. 

(2)

1.5
What is the output voltage (
[image: image108.emf]) for the circuit below?
[image: image20.wmf]

A) –5 Volts 

B) 5 Volts 

C) +12 Volts 

D) –12 Volts 

(2)

1.6
What is the pulse width of the output pulse for the circuit below?
[image: image21.png]




A) 
[image: image2.wmf]41ms


B) 
[image: image3.wmf]41

μs


C) 
[image: image4.wmf]82

μs


D) 
[image: image5.wmf]20.5

μs


(2)

1.7
What is the binary equivalent of E1?

A) 1010 1000 

B) 1110 0001 

C) 1001 0010 

D) 1100 1001 

(2)

1.8
Which logic gate will provide a logic low on the output only when both inputs are high?


A) AND gate 

B) NAND gate 
C) XOR gate 

D) OR gate 

(2)

1.9
What logic function is performed by the circuit shown below?

[image: image22.wmf]0

V



A) AND  

B) NAND  

C) XOR  

D) OR  

(2)

1.10
Which Boolean expression is valid for the circuit shown below?

[image: image23.wmf]1

J





A) 
[image: image6.wmf]FABC

=+




B) 
[image: image7.wmf]FABC

=+

  




C) 
[image: image8.wmf]FABC

=+

  


D)
[image: image9.wmf]FABC

=+

  

(2)
1.11
The Boolean expression 
[image: image10.wmf]FCDE

=++

 simplifies to:


A) 
[image: image11.wmf]FCDE

=++


B) 
[image: image12.wmf]FCDE

=++

  
C) 
[image: image13.wmf]FCDE

=

  

D)
[image: image14.wmf]FCDE

=

  

(2)

Question 2

2.1
Examine the circuit diagram in Fig. 1. The transistor is a Si transistor with a beta value of 120.  Answer the following questions:  


(i) 
Calculate
[image: image15.wmf]B

I

 
(5)


(ii) 
Calculate
[image: image16.wmf]CE

V

 
(2)


(iii) 
What is the function of the capacitor 
[image: image17.wmf]1

C

? 
(2)

[image: image24.wmf]C


2.2
You have been approached by a friend who wants to pump water from a river into his dam.  In the dam there is a level switch. The level switch closes when the water level is low and open when the water level is hi. The switch can only handle a maximum current of 500 mA.   The motor driving the pump is a 90 W / 36 V motor. Design a circuit that will turn the motor on and off.  In your design you must use a 2N3050A transistor with 
[image: image18.wmf]100

b

=

. 


(5)

Question 3
3.1
Draw a circuit diagram for an inverting amplifier. 

(3)

3.1
Consider Fig. 2 and proof that the output voltage of the circuit in is given by: 

[image: image19.wmf]0

in

dV

VRC

dt

=-


 (7)

[image: image25.wmf]75F

m


Question 4
4.1
Examine the circuit in Fig. 3 and draw the expected voltage vs. time graph for pin 2 and pin 3 of the 555.  The graphs must be drawn in phase.
(4)

[image: image26.wmf]500

W


� EMBED MSPhotoEd.3  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





Inside the 555





Fig. 2























� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���








Fig. 3





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





Inside the 555





� EMBED MSPhotoEd.3  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.3  ���





� EMBED Equation.3  ���





� EMBED Equation.3  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





Fig. 1





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.DSMT4  ���





� EMBED Equation.3  ���





� EMBED Equation.3  ���





� EMBED Equation.3  ���





� EMBED Equation.3  ���





� EMBED Equation.DSMT4  ���








[image: image27.wmf]9V

-

[image: image28.wmf]1k

W



[image: image29.wmf]12V

[image: image30.wmf]3V

[image: image31.wmf]2V

[image: image32.wmf]o

V

[image: image33.wmf]2

R

[image: image34.wmf]in

V

[image: image35.wmf]0

V

[image: image36.wmf]9V

+

[image: image37.wmf]C

[image: image38.wmf]1k

W

[image: image39.wmf]9V

[image: image40.png]


[image: image41.wmf]555

[image: image42.wmf]0

V

[image: image43.wmf]12

,0.1

μF

CC

=

[image: image44.wmf]2

C

[image: image45.wmf]20V

cc

V

=+

[image: image46.wmf]R

[image: image47.wmf]9V

[image: image48.wmf]C

[image: image49.wmf]1

R

[image: image50.wmf]2k

W

[image: image51.wmf]12V

-

[image: image52.wmf]12V

+

[image: image53.wmf]5k

W

[image: image54.wmf]K

[image: image55.wmf]F

[image: image56.wmf]B

[image: image57.wmf]A

[image: image58.wmf]F

[image: image59.wmf]B

[image: image60.wmf]A

[image: image61.wmf]4.7k

A

R

W

[image: image62.wmf]1

C

[image: image63.wmf]680k

B

R

W

[image: image64.wmf]120

b

=

[image: image65.wmf]555

[image: image66.emf][image: image67.wmf]2V

[image: image68.wmf]o

V

[image: image69.wmf]3V

[image: image70.wmf]12V

+

[image: image71.wmf]12V

-

[image: image72.emf]TRIG

2

OUT

3

RST

4

CVOLT

5

THR

6

DISC

7

VCC

8

GND

1

[image: image73.wmf]555

[image: image74.wmf]1k

W

[image: image75.wmf]120

b

=

[image: image76.wmf]680k

B

R

W

[image: image77.wmf]500

W

[image: image78.wmf]1

C

[image: image79.wmf]4.7k

A

R

W

[image: image80.wmf]1k

W

[image: image81.wmf]75F

m

[image: image82.wmf]1

J

[image: image83.wmf]2k

W

[image: image84.wmf]0

V

[image: image85.wmf]20V

cc

V

=+

[image: image86.png]


[image: image87.emf][image: image88.wmf]2

C

[image: image89.wmf]A

[image: image90.wmf]B

[image: image91.wmf]F

[image: image92.emf][image: image93.wmf]C

[image: image94.wmf]12

,0.1

μF

CC

=

[image: image95.emf]TRIG

2

OUT

3

RST

4

CVOLT

5

THR

6

DISC

7

VCC

8

GND

1

[image: image96.wmf]9V

[image: image97.wmf]5k

W

[image: image98.wmf]1

R

[image: image99.wmf]2

R

[image: image100.wmf]C

[image: image101.emf][image: image102.wmf]9V

-

[image: image103.wmf]9V

+

[image: image104.wmf]R

[image: image105.wmf]12V

[image: image106.wmf]K

[image: image107.wmf]in

V

_1253438013.unknown

_1253447169.unknown

_1253510494.unknown

_1253512648.unknown

_1253513265.unknown

_1253513357.unknown

_1253513378.unknown

_1253513316.unknown

_1253513254.unknown

_1253511939.unknown

_1253511940.unknown

_1253511810.unknown

_1253511901.unknown

_1253511938.unknown

_1253511756.unknown

_1253447292.unknown

_1253506444.unknown

_1253506361.unknown

_1253447202.unknown

_1253446730.unknown

_1253447000.unknown

_1253447135.unknown

_1253446803.unknown

_1253446653.unknown

_1253446710.unknown

_1253446616.unknown

_1253445892.unknown

_1253445954.unknown

_1253438027.unknown

_1253445853.unknown

_1248717280.unknown

_1253437668.unknown

_1253437944.unknown

_1253437979.unknown

_1253437763.unknown

_1253437519.unknown

_1253437625.unknown

_1253436097.unknown

_1253436226.unknown

_1253436272.unknown

_1253436129.unknown

_1253435960.unknown

_1253436072.unknown

_1221531663.unknown

_1248676279.unknown

_1248677002.unknown

_1248676337.unknown

_1221714985.unknown

_1221715006.unknown

_1221531783.unknown

_1221225372.unknown

_1221531541.unknown

_1221225169.unknown

_1221225344.unknown

_1221225266.unknown

_1124452760.bin

_1221225124.unknown

